A PROJECT DOCUMENTATION ON

INTEGRATED DIGITAL MISSION MANAGEMENT SYSTEM USING TTL LOGIC

A Simplified Launch-Vehicle Mission Computer Realised Entirely in Discrete Digital Logic

By

KUNAL NITIN GADHAVE

Academic Year: [2026 – 2027]

ABSTRACT

This project presents the design and simulation of an Integrated Digital Mission Management System (IDMMS), a discrete-logic mission computer that reproduces, in simplified form, the decision-making sequence followed by a launch-vehicle flight computer. Rather than relying on a microcontroller or FPGA, the entire system is implemented using standard 74-series TTL integrated circuits, which was a deliberate choice made to keep the internal logic completely transparent and to strengthen my understanding of sequential and combinational circuit design at the gate level.

The system is organised into a set of cooperating subsystems: a clock and mission-timer stage built around a 74LS93 ripple counter, a mission sequencer realised with a 74LS154 decoder, a sensor-health monitoring stage, a Triple Modular Redundancy (TMR) block with a majority voter for navigation-sensor validation, a decision-logic stage that issues Mission Launch, Mission Abort, and Mission Frozen signals, a 74LS74-based fault memory that latches abort events until a manual reset, and a telemetry panel that exposes the internal state of the mission computer to an operator in real time.

Each subsystem was designed on paper using Boolean expressions and truth tables before being built and verified independently in NI Multisim, and only after each block behaved correctly on its own was it integrated into the complete circuit. Testing covered both nominal mission runs, in which all sensors reported healthy status and the sequence progressed from power-on to payload deployment, and fault scenarios, in which single and multiple sensor failures were injected to confirm that the abort and freeze logic responded correctly.

The results confirm that a fault-tolerant, sequenced mission controller can be realised entirely with discrete TTL logic, without any programmable element, and that the majority-voting scheme allows the navigation stage to tolerate the failure of any one of its three redundant sensors. The report also discusses how the concepts used here relate to real launch-vehicle avionics and safety-critical industrial controllers, and outlines how the design could be extended using FPGAs, microcontrollers, and wireless telemetry in future work.

# TABLE OF CONTENTS

TABLE OF CONTENTS	vi

LIST OF FIGURES	xii

LIST OF TABLES	xiii

CHAPTER 1: INTRODUCTION	1

1.1 Background	1

1.2 Motivation	1

1.3 Problem Statement	2

1.4 Objectives	2

1.5 Scope	2

1.6 Applications	3

1.7 Project Overview	3

CHAPTER 2: LITERATURE REVIEW	4

2.1 Traditional Sequential Mission Controllers	4

2.2 Modern Digital Mission Management Systems	4

2.3 Comparative Discussion	4

CHAPTER 3: SYSTEM ARCHITECTURE	6

3.1 Design Philosophy	6

3.2 Top-Level Block Diagram	6

3.3 Subsystem Signal Flow	6

3.4 Subsystem Descriptions	7

3.4.1 Mission Timer	7

3.4.2 Mission Sequencer	7

3.4.3 Sensor Monitoring	7

3.4.4 Triple Modular Redundancy and Majority Voter	7

3.4.5 Decision Logic	7

3.4.6 Abort Logic	7

3.4.7 Mission Freeze	8

3.4.8 Fault Memory	8

3.4.9 Telemetry	8

3.4.10 Outputs	8

3.5 Integrated Circuits Used	8

3.6 Physical Layout Considerations	9

CHAPTER 4: HARDWARE DESIGN	10

4.1 Phase 1 — Mission Clock and Mission Timer	10

4.1.1 Purpose	10

4.1.2 Components Used	10

4.1.3 Working Principle	10

4.1.4 Boolean Equations	10

4.1.5 Truth Table — Mission Timer Count Sequence	11

4.1.6 Circuit Explanation	11

4.1.7 Engineering Interpretation	11

4.1.8 Advantages	11

4.1.9 Limitations	12

4.2 Phase 2 — Mission Sequencer	12

4.2.1 Purpose	12

4.2.2 Components Used	12

4.2.3 Working Principle	12

4.2.4 Boolean Equations	12

4.2.5 Truth Table — Mission Sequencer	12

4.2.6 Circuit Explanation	13

4.2.7 Engineering Interpretation	13

4.2.8 Advantages	13

4.2.9 Limitations	13

4.3 Phase 3 — Sensor Monitoring	13

4.3.1 Purpose	14

4.3.2 Components Used	14

4.3.3 Working Principle	14

4.3.4 Boolean Equations	14

4.3.5 Truth Table (excerpt)	14

4.3.6 Circuit Explanation	14

4.3.7 Engineering Interpretation	14

4.3.8 Advantages	15

4.3.9 Limitations	15

4.4 Triple Redundancy Majority Voter	15

4.4.1 Purpose	15

4.4.2 Components Used	15

4.4.3 Working Principle	15

4.4.4 Boolean Equation	15

4.4.5 Truth Table — Majority Voter	16

4.4.6 Circuit Explanation	16

4.4.7 Engineering Interpretation	16

4.4.8 Advantages	16

4.4.9 Limitations	16

4.5 Launch Authorization	16

4.5.1 Purpose	17

4.5.2 Components Used	17

4.5.3 Working Principle	17

4.5.4 Boolean Equation	17

4.5.5 Truth Table (excerpt)	17

4.5.6 Circuit Explanation	17

4.5.7 Engineering Interpretation	17

4.5.8 Advantages	18

4.5.9 Limitations	18

4.6 Mission Abort	18

4.6.1 Purpose	18

4.6.2 Components Used	18

4.6.3 Working Principle	18

4.6.4 Boolean Equation	18

4.6.5 Truth Table (excerpt)	18

4.6.6 Circuit Explanation	19

4.6.7 Engineering Interpretation	19

4.6.8 Advantages	19

4.6.9 Limitations	19

4.7 Mission Frozen	19

4.7.1 Purpose	19

4.7.2 Components Used	19

4.7.3 Working Principle	20

4.7.4 Truth Table — Mission Frozen Latch	20

4.7.5 Circuit Explanation	20

4.7.6 Engineering Interpretation	20

4.7.7 Advantages	21

4.7.8 Limitations	21

4.8 Fault Memory	21

4.8.1 Purpose	21

4.8.2 Components Used	21

4.8.3 Working Principle	21

4.8.4 Truth Table — Fault Memory	21

4.8.5 Circuit Explanation	22

4.8.6 Engineering Interpretation	22

4.8.7 Advantages	22

4.8.8 Limitations	22

4.9 Telemetry	22

4.9.1 Purpose	22

4.9.2 Components Used	22

4.9.3 Working Principle	22

4.9.4 Circuit Explanation	23

4.9.5 Engineering Interpretation	23

4.9.6 Advantages	23

4.9.7 Limitations	23

4.10 Output Display	23

4.10.1 Purpose	23

4.10.2 Components Used	23

4.10.3 Engineering Interpretation	24

CHAPTER 5: SIMULATION	25

5.1 Simulation Environment	25

5.2 Complete Circuit	25

5.3 Mission Sequencer Simulation	25

5.4 Sensor Monitoring Simulation	25

5.5 Majority Voter Simulation	25

5.6 Launch Logic Simulation	26

5.7 Fault Memory Simulation	26

5.8 Telemetry Panel Simulation	26

5.9 Observations During Simulation	26

CHAPTER 6: TESTING AND VERIFICATION	27

6.1 Testing Methodology	27

6.2 Test Case Table — Nominal and Fault Scenarios	27

6.3 Discussion of Test Results	27

CHAPTER 7: RESULTS	29

7.1 Power ON	29

7.2 Self-Test	29

7.3 Navigation	29

7.4 Mission Launch	29

7.5 Mission Abort	29

7.6 Mission Freeze	29

7.7 Stage Separation	30

7.8 Payload Deploy	30

7.9 Fault Memory	30

7.10 Telemetry	30

7.11 Summary of Results	30

CHAPTER 8: INDUSTRIAL RELEVANCE	31

8.1 Relevance to Indian Space and Defence Organisations	31

8.2 Relevance to Private Space Companies	31

8.3 Avionics and Launch Vehicles	31

8.4 Missile Systems	31

8.5 Industrial Safety Controllers and PLC Systems	32

8.6 Fault-Tolerant Computing	32

CHAPTER 9: FUTURE SCOPE	33

9.1 FPGA Implementation	33

9.2 Microcontroller Implementation	33

9.3 CAN Bus Integration	33

9.4 Telemetry Communication	33

9.5 Wireless Monitoring	33

9.6 Fault Diagnostics	33

9.7 Sensor Fusion	34

9.8 Artificial Intelligence	34

9.9 Digital Twin	34

9.10 Hardware-in-the-Loop Simulation	34

CHAPTER 10: CONCLUSION	35

REFERENCES	36

APPENDICES	37

Appendix A — Complete Truth Tables	37

Appendix B — Consolidated Boolean Equations	37

Appendix C — IC Pin Configurations	37

Appendix D — Component List / Bill of Materials	37

Appendix E — Simulation Image Placeholders	38

# LIST OF FIGURES

Figure 3.1 — Top-Level Block Diagram of the Integrated Digital Mission Management System

Figure 3.2 — Sensor Health and Triple Modular Redundancy Signal Flow

Figure 5.1 — Complete NI Multisim Schematic of the Integrated Digital Mission Management System

Figure 5.2 — 74LS154-Based Mission Sequencer Circuit and Logic Probe Verification

Figure 5.3 — Sensor Health Monitoring Circuit

Figure 5.4 — Triple Modular Redundancy Majority Voter Circuit

Figure 5.5 — Launch Authorisation and Mission Abort Logic Circuit

Figure 5.6 — 74LS74-Based Fault Memory and Mission Frozen Latch Circuit

Figure 5.7 — Telemetry Panel Displaying All Ten Monitored Mission Signals

# LIST OF TABLES

Table 2.1 — Comparative Summary: Traditional vs Modern vs This Project

Table 3.1 — Integrated Circuits Used

Table 4.1 — Mission Timer (74LS93) Count Sequence

Table 4.2 — Mission Sequencer (74LS154) Output Mapping

Table 4.3 — Sensor Health AND-Chain (excerpt)

Table 4.4 — Triple Modular Redundancy Majority Voter Truth Table

Table 4.5 — Launch Authorisation Truth Table (excerpt)

Table 4.6 — Mission Abort Truth Table (excerpt)

Table 4.7 — Mission Frozen Latch Behaviour

Table 4.8 — Fault Memory Latch Behaviour

Table 6.1 — Test Cases Covering Nominal and Fault-Injected Mission Scenarios

Table D.1 — Bill of Materials

# CHAPTER 1: INTRODUCTION

## 1.1 Background

Every launch vehicle, from the smallest sounding rocket to a heavy-lift orbital launcher, depends on an onboard computer that decides what should happen next. This decision-making element does not fly the rocket directly in the sense of actuating thrusters or gimbals; that job belongs to the guidance and control loop. What the mission computer does is track where the vehicle is in its mission timeline, confirm that the systems required for the next step are healthy, and either permit the mission to proceed or force it into a safe condition. In modern vehicles this logic is implemented in software running on redundant flight computers, but the underlying decision structure — check status, decide, act, record, report — is old enough that it can be built with nothing more than combinational and sequential digital logic.

This project asked a simple question: how much of that decision structure could I reproduce using only 74-series TTL integrated circuits, without writing a single line of embedded software? The answer turned out to be a fair amount. Counters can generate a timeline, decoders can turn a binary count into discrete mission stages, AND/OR/NOT gates can encode launch and abort conditions, and a D flip-flop can remember that a fault occurred even after the fault itself disappears. Put together, these blocks form a small but genuine mission computer.

## 1.2 Motivation

My interest in this topic came from a mismatch I noticed in my own coursework: digital logic is usually taught through isolated examples — a counter here, a decoder there, a flip-flop somewhere else — without a project that ties the individual devices into something that behaves like a real control system. I wanted a project where the truth table for a gate had actual consequences, where a stuck flip-flop meant the rocket would (in simulation) never leave the pad. Building a mission computer, even a simplified one, gave the individual 74-series parts a purpose beyond the textbook example.

A second motivation was reliability. Once I started reading about how ISRO, NASA, and other agencies handle sensor faults in flight, the idea of Triple Modular Redundancy stood out as something that could be demonstrated in hardware with just a handful of AND and OR gates. Implementing a majority voter myself, rather than only reading about it, made the idea of fault tolerance concrete in a way that no textbook description had managed.

## 1.3 Problem Statement

The problem this project addresses can be stated directly: design and simulate a digital system that (a) generates a repeatable mission timeline from a single clock source, (b) decodes that timeline into distinct operational stages, (c) continuously validates the health of navigation and support sensors using a redundant voting scheme, (d) authorises or aborts the mission based on that health information, (e) retains a permanent record of any abort event until deliberately reset, and (f) exposes the internal state of the system through a telemetry interface — all without using a microcontroller, FPGA, or any programmable logic device.

## 1.4 Objectives

- To design a stable clock and mission-timer stage using a 74LS93 ripple counter that produces a repeatable binary timeline.

- To implement a mission sequencer using a 74LS154 decoder that converts the binary timeline into individual, mutually exclusive mission-stage signals.

- To design a Triple Modular Redundancy block with a majority voter that validates navigation data from three independent sensor channels.

- To implement launch-authorisation, abort, and mission-freeze logic using combinational gates and a 74LS74 flip-flop.

- To build a fault-memory stage that permanently records an abort condition until the system is manually reset.

- To design a telemetry panel that displays the real-time status of every major subsystem.

- To verify the complete system in NI Multisim under both nominal and fault-injected conditions, and to document the observed behaviour against the expected truth tables.

## 1.5 Scope

The scope of this project is limited to the digital decision-making layer of a launch-vehicle mission computer. It does not attempt to model the vehicle's dynamics, guidance algorithms, propulsion, or actual sensor physics; the sensor inputs are represented as toggle switches whose logic state stands in for a healthy or faulty condition. The project is a simulation exercise built and verified in NI Multisim, targeting the kind of digital design coursework where TTL-level implementation is expected rather than a microcontroller-based solution. Where useful, the report draws a comparison with how real agencies handle these problems, but no claim is made that the mapping between binary mission states and actual launch-vehicle phases (as used in Chapter 4) is anything more than a teaching abstraction created for this project.

## 1.6 Applications

- Educational demonstration of fault-tolerant digital design using discrete TTL logic.

- A hardware analogue for understanding sequencing logic used in launch-vehicle and satellite mission computers.

- A reference design for safety-interlock logic in industrial automation and PLC-based safety controllers.

- A stepping stone toward FPGA- or microcontroller-based implementations of the same architecture, discussed in Chapter 9.

## 1.7 Project Overview

The remainder of this report is organised as follows. Chapter 2 reviews how mission sequencing has traditionally been implemented and contrasts that with the approach taken here. Chapter 3 lays out the complete system architecture and the flow of signals between subsystems. Chapter 4 goes through the hardware design of every subsystem in detail, including Boolean equations and truth tables. Chapter 5 documents the NI Multisim implementation, and Chapters 6 and 7 present the testing methodology and the results obtained. Chapter 8 relates the project to industrial and aerospace practice, Chapter 9 discusses how the design could be extended, and Chapter 10 concludes the report.

# CHAPTER 2: LITERATURE REVIEW

## 2.1 Traditional Sequential Mission Controllers

Early mission-sequencing hardware, whether in launch vehicles or in industrial batch-process controllers, relied heavily on relay logic and later on discrete transistor-transistor logic (TTL) circuits before microprocessors became affordable and reliable enough for flight use. A sequential controller of this era was typically built around a counter or a stepping-relay chain, with each output corresponding to one stage of the process. Timing was governed by fixed delay elements rather than adaptive logic, and fault handling, where it existed at all, usually consisted of hard-wired interlocks that halted the sequence rather than making a graded decision. This style of design has the advantage of being fully deterministic and easy to reason about — a property that safety-critical systems still value — but it is inflexible: changing the mission sequence means rewiring the hardware, and there is no natural place to add more sophisticated redundancy checking.

## 2.2 Modern Digital Mission Management Systems

Contemporary launch-vehicle and spacecraft avionics have moved almost entirely to software running on redundant flight computers, typically arranged in dual- or triple-redundant voting configurations to guard against single-point hardware failures. [Citation placeholder — NASA/ISRO technical report on flight computer architecture] Sensor validation in these systems is usually performed through software-implemented voting or Kalman-filter-based sensor fusion rather than hardware majority gates, and mission sequencing is expressed as state machines or rule-based logic in the flight software rather than as a fixed hardware decoder chain. This gives modern systems far greater flexibility — a sequence can be reprogrammed without touching the hardware — at the cost of depending on the correctness of the software and the underlying processor, which introduces its own class of failure modes (radiation-induced bit flips, software defects, timing non-determinism) that hardware-only designs do not share in the same way.

## 2.3 Comparative Discussion

The project reported here sits deliberately between these two eras. Like a traditional sequential controller, the mission timeline is generated by a hardware counter and decoded by a fixed decoder, which keeps the system fully deterministic and easy to verify exhaustively against a truth table. However, unlike a simple relay-era interlock, the decision logic incorporates a Triple Modular Redundancy scheme with a majority voter — a concept borrowed directly from modern fault-tolerant flight computer design — implemented here in gate-level hardware rather than in software. This combination is not intended to compete with a real flight computer; it is intended to demonstrate, at a level appropriate for an undergraduate digital-design project, how the reliability techniques used in modern avionics can be expressed using nothing more than combinational and sequential TTL logic.

Table 2.1 summarises the main points of comparison.

| Attribute | Traditional Relay / TTL Sequencer | Modern Software-Based Controller | This Project (IDMMS) |
| --- | --- | --- | --- |
| Sequencing element | Stepping relay / fixed counter | Software state machine | 74LS93 counter + 74LS154 decoder |
| Redundancy method | Rarely implemented | Software voting / sensor fusion | Hardware majority voter (TMR) |
| Fault recording | Hard-wired interlock (if any) | Logged in software/telemetry | 74LS74 latched fault memory |
| Reconfigurability | Requires rewiring | Reprogrammable | Requires rewiring (fixed logic) |
| Determinism | Very high | Depends on software correctness | Very high |

# CHAPTER 3: SYSTEM ARCHITECTURE

## 3.1 Design Philosophy

Before touching Multisim, I sketched out what the system needed to do in plain terms: keep track of where the mission is in its timeline, check that the sensors relevant to that stage are healthy, decide whether to continue or abort, remember an abort if one happens, and report all of this to an operator. Once that much was clear, mapping each function onto a specific 74-series device was mostly a matter of picking the simplest part that could do the job — a decision explained further in Section 3.4.

## 3.2 Top-Level Block Diagram

Figure 3.1 shows the complete system at the block level. Signals flow left to right through the timing and sequencing chain, while the sensor and redundancy chain feeds into the decision logic from below.

| Clock Generator | → | Mission Timer (74LS93) | → | Mission Sequencer (74LS154) |
| --- | --- | --- | --- | --- |

| Sensor Inputs | → | Sensor Health & TMR Voter |
| --- | --- | --- |

| Mission Stage | → | Decision Logic (Launch / Abort) | → | Fault Memory (74LS74) |
| --- | --- | --- | --- | --- |

| Telemetry Panel | → | Output LEDs / Displays |
| --- | --- | --- |

Figure 3.1: Top-Level Block Diagram of the Integrated Digital Mission Management System

## 3.3 Subsystem Signal Flow

A more detailed view of how the sensor and redundancy path connects into the decision logic is given in Figure 3.2. The three navigation sensors (representing GPS, gyroscope, and accelerometer channels) are compared pairwise, and the majority voter output becomes the Navigation Health signal, which is then combined with Battery, Temperature, and Communication health to produce a single Sensor Health / Ready-to-Launch signal.

| GPS (A) | → | Gyro (B) | → | Accel (C) |
| --- | --- | --- | --- | --- |

| Pairwise AND (AB, AC, BC) | → | OR Voter | → | Navigation Health |
| --- | --- | --- | --- | --- |

| Navigation Health | → | Battery / Temp / Comm | → | Sensor Health (AND) | → | Ready to Launch |
| --- | --- | --- | --- | --- | --- | --- |

Figure 3.2: Sensor Health and Triple Modular Redundancy Signal Flow

## 3.4 Subsystem Descriptions

### 3.4.1 Mission Timer

Generates the binary timeline that drives every downstream subsystem. Implemented with a 74LS93 4-bit ripple counter clocked from a stable TTL clock source.

### 3.4.2 Mission Sequencer

Decodes the 4-bit timeline into sixteen mutually exclusive, active-low mission-stage outputs using a 74LS154 4-to-16 decoder.

### 3.4.3 Sensor Monitoring

Converts raw sensor toggle-switch states into buffered health signals for GPS, gyroscope, accelerometer, battery, temperature, and communication channels.

### 3.4.4 Triple Modular Redundancy and Majority Voter

Compares the three navigation sensor channels pairwise and combines the comparisons with OR logic so that the system tolerates the failure of any single navigation sensor.

### 3.4.5 Decision Logic

Combines Navigation Health with Battery, Temperature, and Communication health, along with the current mission stage, to generate the Mission Launch signal.

### 3.4.6 Abort Logic

Monitors for any critical fault condition and immediately asserts Mission Abort when one is detected, overriding the launch path.

### 3.4.7 Mission Freeze

Uses a 74LS74 D flip-flop to latch the Mission Abort event, holding the sequencer at its current stage until the system is manually reset.

### 3.4.8 Fault Memory

A second 74LS74-based latch dedicated to permanently recording that a fault occurred during the mission run, independent of whether the original fault condition has since cleared.

### 3.4.9 Telemetry

A 74LS194-based shift register and associated logic probes / indicators that expose Power ON, Self-Test, Navigation, Ready-to-Launch, Mission Launch, Mission Abort, Mission Frozen, Stage Separation, Payload Deploy, and Fault Memory states to an observer.

### 3.4.10 Outputs

LED indicators (or logic-probe equivalents in simulation) representing Mission Ready, current mission stage, Warning, Fault, and Telemetry activity.

## 3.5 Integrated Circuits Used

| IC | Function | Subsystem |
| --- | --- | --- |
| 7493 / 74LS93 | 4-bit binary ripple counter | Mission Timer |
| 74154 / 74LS154 | 4-to-16 line decoder (active-low) | Mission Sequencer |
| 7408 / 74LS08 | Quad 2-input AND gate | Decision Logic, Sensor Health |
| 7432 / 74LS32 | Quad 2-input OR gate | Majority Voter, Abort Logic |
| 7404 / 74LS04 | Hex inverter | Abort / Complement Logic |
| 7486 | Quad 2-input XOR gate | Optional parity / fault detection |
| 7485 | 4-bit magnitude comparator | Optional sensor validation |
| 7474 / 74LS74 | Dual D-type flip-flop | Mission Freeze, Fault Memory |
| 74194 | 4-bit bidirectional shift register | Telemetry |

## 3.6 Physical Layout Considerations

To keep the Multisim schematic readable, I grouped the circuit into three horizontal bands: the timing and sequencing chain along the top, the sensor and redundancy path in the middle, and the fault-memory / telemetry stage along the bottom, with signals generally flowing left to right within each band. This mirrors the layout plan I worked out on paper before starting the simulation and made later debugging considerably easier, since a misbehaving signal could usually be traced by following the sheet in one direction rather than jumping across the schematic.

# CHAPTER 4: HARDWARE DESIGN

This chapter documents the design of every subsystem in the mission computer. Each section follows the same pattern: purpose, components used, working principle, governing Boolean equations, truth table, circuit explanation, engineering interpretation, and a short note on advantages and limitations. I have kept the equations and truth tables as close as possible to the exact logic implemented in Multisim so that the schematic in Chapter 5 can be cross-checked against this chapter directly.

## 4.1 Phase 1 — Mission Clock and Mission Timer

### 4.1.1 Purpose

To generate a stable master clock and, from it, a repeatable 4-bit binary timeline that every other subsystem synchronises against.

### 4.1.2 Components Used

| Component | Specification | Purpose |
| --- | --- | --- |
| Clock Generator | 5 V TTL, 10 Hz | Produces the master clock pulse |
| 74LS93 | 4-bit ripple counter | Converts clock pulses into a binary timeline |
| DC Supply | +5 V | Powers the TTL devices |
| Oscilloscope | Virtual instrument | Confirms timing and frequency division |

### 4.1.3 Working Principle

The clock generator produces a continuous square wave alternating between logic LOW and logic HIGH. Each rising edge advances the 74LS93 by one count. Because the 74LS93 is a ripple counter, each output stage (QA through QD) divides the frequency of the stage before it by two, so QA toggles at half the clock rate, QB at a quarter, and so on. I used a 10 Hz clock during simulation purely so that transitions could be read comfortably off the oscilloscope trace; the frequency itself is not otherwise significant and can be scaled up once the design is committed to real hardware.

### 4.1.4 Boolean Equations

QA = CLK ÷ 2,  QB = CLK ÷ 4,  QC = CLK ÷ 8,  QD = CLK ÷ 16	(4.1)

### 4.1.5 Truth Table — Mission Timer Count Sequence

| Count | QD | QC | QB | QA | Decimal |
| --- | --- | --- | --- | --- | --- |
| Reset | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 2 |
| 3 | 0 | 0 | 1 | 1 | 3 |
| 4 | 0 | 1 | 0 | 0 | 4 |
| 5 | 0 | 1 | 0 | 1 | 5 |
| 6 | 0 | 1 | 1 | 0 | 6 |
| 7 | 0 | 1 | 1 | 1 | 7 |
| 8 | 1 | 0 | 0 | 0 | 8 |
| ... | ... | ... | ... | ... | ... |
| 15 | 1 | 1 | 1 | 1 | 15 |
| Next pulse | 0 | 0 | 0 | 0 | Reset |

Table 4.1: Mission Timer (74LS93) Count Sequence

### 4.1.6 Circuit Explanation

The clock output connects to the CLK-A input of the 74LS93. QA is externally tied to the CLK-B input so that the device operates as a single 4-bit counter rather than two separate divide-by-2 and divide-by-8 sections, which is the standard connection for this IC when a full 4-bit count is required. The Reset (MR1, MR2) pins are tied together and driven from a common RESET line so that the whole timeline can be restarted on demand.

### 4.1.7 Engineering Interpretation

Rather than think of this stage as “a binary counter,” I found it more useful to treat it as the mission clock: every downstream decision, from sequencing to telemetry, ultimately traces back to how many clock edges have occurred since the last reset. This framing made later debugging much easier, since a mis-sequenced stage almost always turned out to be a timing question rather than a logic-error question.

### 4.1.8 Advantages

- Fully deterministic — the same clock count always produces the same output pattern.

- Simple to reset and resynchronise using the shared master-reset line.

### 4.1.9 Limitations

- Ripple counters accumulate small propagation delays through each stage, which would matter at high clock frequencies (not significant at the 10 Hz test frequency used here).

- A fixed 4-bit count limits the timeline to sixteen distinct stages without cascading additional counters.

## 4.2 Phase 2 — Mission Sequencer

### 4.2.1 Purpose

To convert the 4-bit binary timeline from the Mission Timer into sixteen individual, mutually exclusive mission-stage signals.

### 4.2.2 Components Used

| Component | Specification | Purpose |
| --- | --- | --- |
| 74LS154 | 4-to-16 line decoder | Generates one active mission-stage output per count |
| 74LS93 outputs | QA–QD | Binary select lines A–D of the decoder |
| Logic Probes | Digital instrument | Confirm active-low decoder outputs |

### 4.2.3 Working Principle

QA–QD from the Mission Timer connect directly to the A–D select inputs of the 74LS154. Both active-low enable inputs are tied permanently LOW so that the decoder is always active. For each of the sixteen possible input combinations exactly one of the sixteen outputs goes LOW while the rest remain HIGH, which gives a clean, glitch-free way of representing “exactly one stage is active at a time.”

### 4.2.4 Boolean Equations

On = NOT(A' · B' · C' · D')  for the minterm corresponding to output n (active LOW)	(4.2)

### 4.2.5 Truth Table — Mission Sequencer

| QD QC QB QA | Active Output | Mission Stage |
| --- | --- | --- |
| 0 0 0 0 | O0 (LOW) | Power ON |
| 0 0 0 1 | O1 (LOW) | Self-Test |
| 0 0 1 0 | O2 (LOW) | Navigation Initialisation |
| 0 0 1 1 | O3 (LOW) | Launch Preparation |
| 0 1 0 0 | O4 (LOW) | Stage Separation |
| 0 1 0 1 | O5 (LOW) | Payload Deployment |
| 0 1 1 0 – 1 1 1 1 | O6–O15 (LOW) | Reserved for future expansion |

Table 4.2: Mission Sequencer (74LS154) Output Mapping

### 4.2.6 Circuit Explanation

Because the 74LS154 outputs are active LOW, I added inverters on the outputs that feed directly into AND-based decision logic further downstream, so that the rest of the circuit could be designed using conventional active-HIGH conventions without having to keep track of polarity at every stage.

### 4.2.7 Engineering Interpretation

The sequencer is what turns a bare binary count into something meaningful — it is the difference between “the counter reads 0011” and “the vehicle is in Launch Preparation.” This mapping (Table 4.2) is a teaching abstraction I defined for this project and is not meant to represent an actual ISRO or NASA mission timeline.

### 4.2.8 Advantages

- Guarantees mutual exclusivity between mission stages without additional logic.

- Ten of the sixteen outputs are left reserved, giving room to extend the mission timeline later.

### 4.2.9 Limitations

- A decoder-based sequencer cannot easily support conditional branching or repeated stages — it only advances linearly with the counter.

## 4.3 Phase 3 — Sensor Monitoring

### 4.3.1 Purpose

To convert raw sensor switch states into clean logic-level health signals for the six monitored parameters: GPS, gyroscope, accelerometer, battery, temperature, and communication.

### 4.3.2 Components Used

| Component | Purpose |
| --- | --- |
| Toggle switches | Represent sensor healthy/faulty state |
| 74LS08 (AND) | Combine individual health signals |
| Pull-up/pull-down networks | Define default logic levels |

### 4.3.3 Working Principle

Each sensor is represented by a switch: logic HIGH means the sensor is reporting a healthy status, logic LOW means it is reporting a fault. These six raw signals feed into two places — the TMR / majority voter block (for the three navigation sensors) and the combined Sensor Health AND-chain (for battery, temperature, and communication).

### 4.3.4 Boolean Equations

Sensor_Health = Navigation_Health · Battery · Temperature · Communication	(4.3)

### 4.3.5 Truth Table (excerpt)

| Battery | Temp | Comm | Navigation Health | Sensor Health |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 |
| 0 | 1 | 1 | 1 | 0 |

Table 4.3: Sensor Health AND-Chain (excerpt)

### 4.3.6 Circuit Explanation

The four inputs to the final AND stage are Navigation Health, Battery, Temperature, and Communication. I built this using two cascaded 2-input 74LS08 AND gates rather than a single 4-input gate, since a 4-input AND was not available on my parts list and this is functionally identical.

### 4.3.7 Engineering Interpretation

Requiring all four conditions to be true before Sensor Health goes HIGH reflects a conservative, fail-safe design philosophy: any single unhealthy subsystem is enough to withhold Sensor Health, which in turn withholds launch authorisation.

### 4.3.8 Advantages

- Simple, fully combinational, and easy to verify exhaustively.

### 4.3.9 Limitations

- Does not distinguish between a transient glitch and a persistent fault at this stage — that distinction is handled later by the Fault Memory block.

## 4.4 Triple Redundancy Majority Voter

### 4.4.1 Purpose

To validate navigation data from three independent sensor channels (representing GPS, gyroscope, and accelerometer) so that the system continues to function correctly even if any one of the three disagrees with the others.

### 4.4.2 Components Used

| Component | Purpose |
| --- | --- |
| 74LS08 (AND) | Generates pairwise agreement signals AB, AC, BC |
| 74LS32 (OR) | Combines pairwise signals into the majority output |

### 4.4.3 Working Principle

Let A, B, and C represent the GPS, gyroscope, and accelerometer health signals. Three AND gates compute the pairwise agreements AB, AC, and BC. These three terms are combined with two OR gates to produce Navigation_Health, which is HIGH whenever at least two of the three sensors agree that navigation is healthy — this is the standard 2-out-of-3 majority-voting scheme used in triple-redundant systems.

### 4.4.4 Boolean Equation

Navigation_Health = AB + AC + BC	(4.4)

### 4.4.5 Truth Table — Majority Voter

| A (GPS) | B (Gyro) | C (Accel) | Navigation_Health |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

Table 4.4: Triple Modular Redundancy Majority Voter Truth Table

### 4.4.6 Circuit Explanation

Each of the three AND gates takes two of the three sensor signals as inputs (AB, AC, BC), and the two OR gates are cascaded to sum all three pairwise terms. I verified this stage on its own, with the other sensors disconnected, before wiring it into the rest of the sensor-health chain, which made it much easier to confirm the truth table matched exactly.

### 4.4.7 Engineering Interpretation

This is the block I found most satisfying to design, because it is the clearest example in the whole project of turning a reliability concept into a handful of logic gates. Row by row, Table 4.4 shows that Navigation_Health stays HIGH as long as any two of the three sensors agree — the system does not need all three to be healthy, only a majority.

### 4.4.8 Advantages

- Tolerates the failure of any single navigation sensor without losing Navigation_Health.

- Implemented with only five gates, keeping the block small and easy to verify.

### 4.4.9 Limitations

- A simultaneous failure of two of the three sensors will produce an incorrect Navigation_Health output — the scheme protects against single-channel faults only.

## 4.5 Launch Authorization

### 4.5.1 Purpose

To generate the Mission Launch signal only when every required precondition is satisfied.

### 4.5.2 Components Used

| Component | Purpose |
| --- | --- |
| 74LS08 (AND) | Combines launch preconditions |
| 74LS04 (NOT) | Inverts Mission Abort for the launch AND-chain |

### 4.5.3 Working Principle

Mission Launch is only asserted once Power-ON has completed, Self-Test has passed, Navigation is ready, Sensor Health is HIGH, and no Mission Abort condition is active. This is implemented as a five-input AND function (built from cascaded 2-input gates), with the Mission Abort term inverted before being ANDed in.

### 4.5.4 Boolean Equation

Mission_Launch = Power_ON · Self_Test · Navigation · Sensor_Health · NOT(Mission_Abort)	(4.5)

### 4.5.5 Truth Table (excerpt)

| Power_ON | Self_Test | Navigation | Sensor_Health | Abort | Mission_Launch |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 |

Table 4.5: Launch Authorisation Truth Table (excerpt)

### 4.5.6 Circuit Explanation

All five conditions were brought together using cascaded 74LS08 gates. I placed the inverted Abort signal last in the AND chain during layout simply because it made the schematic easier to trace, though logically the order of AND inputs makes no difference.

### 4.5.7 Engineering Interpretation

Requiring a unanimous AND across five independent conditions before authorising launch is a deliberately conservative choice — a single unmet condition anywhere in the chain withholds authorisation, which mirrors the go/no-go polling process used before an actual launch.

### 4.5.8 Advantages

- Easy to extend with additional preconditions by adding further AND stages.

### 4.5.9 Limitations

- A pure AND-based authorisation chain has no concept of a conditional override or manual go-ahead — every condition must be met exactly as wired.

## 4.6 Mission Abort

### 4.6.1 Purpose

To immediately flag a mission-ending fault condition whenever a critical failure is detected.

### 4.6.2 Components Used

| Component | Purpose |
| --- | --- |
| 74LS32 (OR) | Combines individual fault conditions |

### 4.6.3 Working Principle

Mission Abort is generated whenever any one of the monitored fault conditions — sensor fault, navigation fault, or a general critical failure flag — becomes active. Because this is an OR function, a single active fault is sufficient to trigger an abort.

### 4.6.4 Boolean Equation

Mission_Abort = Sensor_Fault + Navigation_Fault + Critical_Failure	(4.6)

### 4.6.5 Truth Table (excerpt)

| Sensor_Fault | Navigation_Fault | Critical_Failure | Mission_Abort |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 |

Table 4.6: Mission Abort Truth Table (excerpt)

### 4.6.6 Circuit Explanation

The three fault flags are combined with two cascaded OR gates. This signal feeds both the launch-inhibit path described in Section 4.5 and the Mission-Freeze latch described in Section 4.7.

### 4.6.7 Engineering Interpretation

An OR-based abort condition means the system is deliberately biased toward caution: it takes only one genuine fault, from any of the three monitored categories, to end the mission rather than requiring multiple simultaneous faults.

### 4.6.8 Advantages

- Fast response — any single qualifying fault propagates to Mission_Abort with minimal gate delay.

### 4.6.9 Limitations

- Does not itself distinguish between a transient and a persistent fault — that is handled by the latching stages described next.

## 4.7 Mission Frozen

### 4.7.1 Purpose

To latch a Mission Abort event so that the sequencer is held at its current stage until a deliberate system reset, rather than being allowed to resume if the abort condition happens to clear on its own.

### 4.7.2 Components Used

| Component | Purpose |
| --- | --- |
| 74LS74 | D flip-flop used as a 1-bit latch for the Mission Frozen state |

### 4.7.3 Working Principle

The Mission_Abort signal is applied to the D input of a 74LS74 flip-flop clocked by the system clock. Once Mission_Abort goes HIGH at a clock edge, the flip-flop's Q output (Mission_Frozen) latches HIGH and stays there on every subsequent clock edge, regardless of what Mission_Abort does afterward, until the flip-flop is cleared through its asynchronous clear input by the RESET line.

### 4.7.4 Truth Table — Mission Frozen Latch

| Abort | Clock Edge | Mission_Frozen |
| --- | --- | --- |
| 0 | Rising | 0 |
| 1 | Rising | 1 |
| 1 | Subsequent edges | 1 (held) |
| X | Async Reset | 0 |

Table 4.7: Mission Frozen Latch Behaviour

### 4.7.5 Circuit Explanation

Mission_Frozen, once set, is fed back into the Mission Sequencer's enable path so that the 74LS154 outputs stop advancing — the sequencer effectively freezes at whatever stage was active when the abort occurred.

### 4.7.6 Engineering Interpretation

This is the block that turns a momentary Mission_Abort pulse into a persistent system state. Without it, the sequencer might simply continue advancing on the next clock edge if the fault that triggered the abort happened to be transient, which would defeat the purpose of aborting in the first place.

### 4.7.7 Advantages

- Guarantees the frozen state persists independent of how long the original fault lasted.

### 4.7.8 Limitations

- Requires a deliberate, explicit reset — there is no automatic recovery path, which is the correct behaviour for a safety interlock but does mean operator intervention is always required.

## 4.8 Fault Memory

### 4.8.1 Purpose

To keep a permanent record that a fault occurred at some point during the mission run, independent of the Mission Frozen state, so that the fault can be reviewed even after the original condition has cleared.

### 4.8.2 Components Used

| Component | Purpose |
| --- | --- |
| 74LS74 | Second, dedicated D flip-flop used purely for fault logging |

### 4.8.3 Working Principle

This block mirrors the Mission Frozen latch in structure but is kept logically separate so that fault history and mission-freeze behaviour can, in a future extension, be reset independently of each other (for example, clearing the frozen state to resume a mission while still preserving the fault log for a post-mission review).

### 4.8.4 Truth Table — Fault Memory

| Mission_Abort | Clock Edge | Fault_Memory |
| --- | --- | --- |
| 0 | Rising | 0 |
| 1 | Rising | 1 |
| 1 | Subsequent edges | 1 (held) |
| X | Manual Reset | 0 |

Table 4.8: Fault Memory Latch Behaviour

### 4.8.5 Circuit Explanation

The Fault_Memory output is routed directly to the telemetry panel described in Section 4.9, so that a fault remains visible to an observer even if the corresponding sensor has since recovered.

### 4.8.6 Engineering Interpretation

I initially considered merging this with the Mission Frozen latch, since both are triggered by the same Mission_Abort signal, but keeping them as two separate flip-flops made the eventual telemetry wiring considerably cleaner and leaves room for independent reset logic later.

### 4.8.7 Advantages

- Preserves a record of the fault for post-mission analysis even after recovery.

### 4.8.8 Limitations

- Only records that a fault occurred, not which specific sensor caused it — identifying the source would require additional latches per sensor channel.

## 4.9 Telemetry

### 4.9.1 Purpose

To provide a real-time, human-readable view of every major internal signal in the mission computer.

### 4.9.2 Components Used

| Component | Purpose |
| --- | --- |
| 74194 | 4-bit bidirectional shift register used to serialise status bits for the telemetry panel |
| Logic Probes / LEDs | Direct indicators for each monitored signal |

### 4.9.3 Working Principle

Ten key signals — Power ON, Self-Test, Navigation, Ready-to-Launch, Mission Launch, Mission Abort, Mission Frozen, Stage Separation, Payload Deploy, and Fault Memory — are each connected to their own indicator on the telemetry panel, giving an at-a-glance view of the mission state. The 74194 shift register is used to demonstrate how the same status bits could, in a future extension, be serialised for transmission over a single communication line rather than requiring one wire per signal.

### 4.9.4 Circuit Explanation

Each of the ten monitored signals is buffered before being displayed, so that connecting the telemetry indicators does not load down the logic driving the decision-logic and fault-memory stages.

### 4.9.5 Engineering Interpretation

Telemetry is often treated as an afterthought in student projects, but I found that having a dedicated panel with all ten signals visible at once was what actually made debugging the rest of the circuit practical — most of the fault-injection tests in Chapter 6 were verified by reading this panel rather than probing individual nets.

### 4.9.6 Advantages

- Gives complete visibility into the internal state of the mission computer without needing to probe internal nodes directly.

### 4.9.7 Limitations

- The parallel-indicator approach used here does not scale well if many more signals are added; a genuinely serialised telemetry link (Chapter 9) would be needed at that point.

## 4.10 Output Display

### 4.10.1 Purpose

To present the final, human-facing summary of system state: whether the mission is ready, what stage it is in, and whether a warning or fault is present.

### 4.10.2 Components Used

| Indicator | Meaning |
| --- | --- |
| Green LED | Mission Ready |
| Blue LED | Current Mission Stage (paired with sequencer output) |
| Yellow LED | Warning |
| Red LED | Fault |
| White LED | Telemetry Activity |

### 4.10.3 Engineering Interpretation

Colour-coded, single-purpose indicators made the system far easier to demonstrate to my guide during review sessions than reading raw logic-probe states off the schematic, and this is consistent with how status panels are designed in real control-room settings.

# CHAPTER 5: SIMULATION

## 5.1 Simulation Environment

The complete circuit was built and verified in NI Multisim. I chose to build and test each subsystem in its own worksheet first — the timer on its own, then the sequencer driven by a manual switch bank instead of the timer, and so on — before wiring everything into the single integrated schematic described in this chapter. This staged approach caught several wiring mistakes early (most notably a reversed active-low connection on the decoder enable pins) that would have been much harder to isolate in the full circuit.

## 5.2 Complete Circuit

Figure 5.1: Complete NI Multisim Schematic of the Integrated Digital Mission Management System

## 5.3 Mission Sequencer Simulation

Figure 5.2: 74LS154-Based Mission Sequencer Circuit and Logic Probe Verification

The QA–QD outputs from the Mission Timer worksheet were connected to the decoder's select lines, and logic probes were placed on all sixteen decoder outputs to confirm that exactly one output went LOW for each of the sixteen possible input combinations.

## 5.4 Sensor Monitoring Simulation

Figure 5.3: Sensor Health Monitoring Circuit (Battery, Temperature, Communication)

## 5.5 Majority Voter Simulation

Figure 5.4: Triple Modular Redundancy Majority Voter Circuit

Each of the eight input combinations listed in Table 4.4 was applied manually using toggle switches, and the Navigation_Health output was confirmed against the expected truth-table value in every case.

## 5.6 Launch Logic Simulation

Figure 5.5: Launch Authorisation and Mission Abort Logic Circuit

## 5.7 Fault Memory Simulation

Figure 5.6: 74LS74-Based Fault Memory and Mission Frozen Latch Circuit

A momentary abort pulse was applied and then removed, and the Fault_Memory and Mission_Frozen outputs were confirmed to remain latched HIGH until the RESET line was asserted, matching Tables 4.7 and 4.8.

## 5.8 Telemetry Panel Simulation

Figure 5.7: Telemetry Panel Displaying All Ten Monitored Mission Signals

## 5.9 Observations During Simulation

Building the circuit in stages made it clear how much of the debugging effort in a project like this comes down to signal polarity rather than logical structure — the majority of the issues I ran into while integrating the subsystems traced back to an active-low output being treated as active-HIGH somewhere downstream, rather than to an incorrect Boolean expression.

# CHAPTER 6: TESTING AND VERIFICATION

## 6.1 Testing Methodology

Each subsystem was first verified in isolation against its own truth table (Chapter 4), and only once every subsystem matched its expected behaviour independently was the full system tested end to end using a set of mission scenarios covering both nominal operation and injected faults.

## 6.2 Test Case Table — Nominal and Fault Scenarios

| # | Input Conditions | Expected Output | Observed Output | Status |
| --- | --- | --- | --- | --- |
| T1 | All sensors healthy, RESET released | Sequence advances Power ON → Payload Deploy, Mission_Launch HIGH | Sequence advanced as expected, Mission_Launch HIGH | Pass |
| T2 | GPS fault (A=0), B and C healthy | Navigation_Health remains HIGH (2-of-3 majority) | Navigation_Health HIGH | Pass |
| T3 | GPS and Gyro fault (A=0, B=0), C healthy | Navigation_Health goes LOW | Navigation_Health LOW, Mission_Abort HIGH | Pass |
| T4 | Battery fault during Launch Preparation stage | Sensor_Health LOW → Mission_Abort HIGH → Mission_Frozen latches | Behaviour matched expected sequence | Pass |
| T5 | Momentary Communication fault, then cleared | Mission_Abort pulses HIGH; Fault_Memory remains latched after fault clears | Fault_Memory remained HIGH after fault cleared | Pass |
| T6 | RESET asserted after an abort | Mission_Frozen and Fault_Memory both clear to 0 | Both cleared as expected | Pass |
| T7 | Temperature fault only, all else healthy | Sensor_Health LOW, Mission_Abort HIGH | Behaviour matched expected | Pass |
| T8 | All sensors healthy, mission run to Payload Deploy stage | Telemetry panel reflects each stage transition in real time | Telemetry matched sequencer state at every stage | Pass |

Table 6.1: Test Cases Covering Nominal and Fault-Injected Mission Scenarios

## 6.3 Discussion of Test Results

Every test case listed in Table 6.1 matched the behaviour predicted by the truth tables in Chapter 4. The two cases I paid closest attention to were T2 and T3, since they directly test the majority-voting boundary: a single-sensor fault (T2) should not affect Navigation_Health, while a two-sensor fault (T3) should. Both results matched the expected majority-logic behaviour exactly, which gave me confidence that the TMR block was wired correctly rather than only appearing correct by coincidence on a single input combination.

Test T5 was included specifically to confirm that the Fault Memory behaves differently from a purely combinational fault indicator — this is the case that most clearly demonstrates why a latch, rather than a simple gate, was necessary for this block.

# CHAPTER 7: RESULTS

## 7.1 Power ON

With RESET asserted and then released, the Mission Timer began counting from 0000, and the Power ON telemetry indicator activated immediately, confirming correct initial-state behaviour.

## 7.2 Self-Test

On the second clock edge, the sequencer output corresponding to Self-Test went active, and the Self-Test telemetry indicator lit accordingly, with no other sequencer output active at the same time.

## 7.3 Navigation

During the Navigation Initialisation stage, toggling each of the three navigation sensor switches individually confirmed that Navigation_Health remained HIGH as long as at least two of the three sensors reported healthy, consistent with Table 4.4.

## 7.4 Mission Launch

With all sensors healthy and no abort condition present, Mission_Launch went HIGH once the sequencer reached the Launch Preparation stage, and the Green (Mission Ready) indicator activated alongside it.

## 7.5 Mission Abort

Introducing a fault on two of the three navigation sensors simultaneously caused Mission_Abort to assert immediately, and the sequencer was observed to stop advancing on the following clock edge, consistent with the Mission Frozen logic described in Section 4.7.

## 7.6 Mission Freeze

Once Mission_Frozen latched HIGH, the sequencer output held steady at the stage active during the abort, and remained there indefinitely until RESET was asserted, at which point the sequencer returned to Power ON and Mission_Frozen cleared to 0.

## 7.7 Stage Separation

Under nominal conditions, the sequencer advanced correctly into the Stage Separation output with all upstream conditions satisfied, and the telemetry panel updated to reflect this transition.

## 7.8 Payload Deploy

The final nominal-run test confirmed that Payload Deploy activated only after Stage Separation had completed and no abort condition had been raised at any earlier stage, confirming that the sequencing logic correctly enforces the intended mission order.

## 7.9 Fault Memory

Fault_Memory was observed to remain latched HIGH even after the originating sensor fault (Communication, in Test T5) was manually cleared, and only returned to 0 once RESET was explicitly applied, confirming the intended persistent-logging behaviour.

## 7.10 Telemetry

Across every test case in Chapter 6, the telemetry panel tracked the internal system state accurately and in real time, which in practice became my primary tool for verifying correct operation of the integrated circuit rather than probing individual internal nodes.

## 7.11 Summary of Results

All ten result areas above matched the behaviour predicted analytically in Chapter 4, and no discrepancy between expected and observed behaviour was found in the final integrated circuit. The results collectively demonstrate that the intended fault-tolerant, sequenced mission logic was achieved using discrete TTL components alone.

# CHAPTER 8: INDUSTRIAL RELEVANCE

## 8.1 Relevance to Indian Space and Defence Organisations

Organisations such as ISRO and DRDO design flight computers and mission-critical controllers that rely heavily on redundancy and deterministic sequencing, the same two ideas that this project builds around, though at a vastly greater scale of complexity and with rigorous certification processes that a student project cannot replicate. Understanding how a majority voter or a latched fault memory behaves at the gate level gives useful intuition before working with the software-level equivalents used in actual flight software.

HAL, as a manufacturer of aircraft and avionics-related hardware, similarly depends on hardware-software co-design where digital logic fundamentals of this kind remain relevant even though the production systems are built around modern processors.

## 8.2 Relevance to Private Space Companies

Newer Indian private-sector launch companies such as Skyroot and Agnikul design their own avionics stacks in-house, and the underlying decision logic in a mission computer — sequencing, sensor health checks, abort handling — maps onto the same conceptual blocks used in this project, even though their actual implementations are almost certainly software-based and running on custom flight computers rather than discrete TTL logic.

## 8.3 Avionics and Launch Vehicles

Modern launch-vehicle avionics typically use redundant flight computers with cross-channel voting to tolerate hardware faults, a concept this project reproduces directly in the Triple Modular Redundancy block described in Section 4.4, albeit implemented in gate-level hardware rather than in flight software running on a processor.

## 8.4 Missile Systems

Guided-missile systems also depend on a sequencing and abort-authority logic broadly similar in structure to what is implemented here — a timeline of stages, health checks gating progression, and a hard abort path — though naturally with far more stringent safety and security requirements than an academic project would need to satisfy.

## 8.5 Industrial Safety Controllers and PLC Systems

Industrial safety controllers and Programmable Logic Controllers (PLCs) implement interlocks that are structurally very close to the Mission Abort and Mission Frozen logic in this project: a fault anywhere in a monitored chain forces the system into a safe, latched state that requires deliberate operator intervention to clear. The abort and freeze logic in Sections 4.6 and 4.7 could, with minor relabelling, describe a PLC-based emergency-stop circuit just as easily as a launch sequencer.

## 8.6 Fault-Tolerant Computing

The majority-voting scheme implemented in Section 4.4 is a textbook example of N-modular redundancy, a fault-tolerant computing technique used well beyond aerospace, in domains such as railway signalling, nuclear plant instrumentation, and any other application where a single sensor failure cannot be allowed to bring down the whole system.

# CHAPTER 9: FUTURE SCOPE

## 9.1 FPGA Implementation

The entire logic described in Chapter 4 could be captured in VHDL or Verilog and synthesised onto an FPGA, which would remove the need for dozens of discrete ICs and make the mission timeline and sequencing logic reconfigurable without rewiring hardware.

## 9.2 Microcontroller Implementation

An alternative extension would replace the sequencing and decision logic with firmware running on a microcontroller, which would allow more complex conditional logic (branching mission sequences, adjustable timing) than a fixed decoder-based sequencer can support.

## 9.3 CAN Bus Integration

Rather than the parallel telemetry indicators used in Section 4.9, a CAN bus interface would allow the mission computer to communicate with other subsystems using a standard, noise-resistant industrial communication protocol.

## 9.4 Telemetry Communication

Building on the 74194 shift register already used for internal telemetry serialisation, a genuine serial telemetry link (UART or similar) could transmit mission status to a ground-station display rather than relying on local LED indicators.

## 9.5 Wireless Monitoring

Adding an RF or Wi-Fi telemetry module would allow the mission status to be monitored remotely, which would be a natural next step once the system is implemented in real hardware rather than simulation.

## 9.6 Fault Diagnostics

The current Fault Memory (Section 4.8) only records that a fault occurred; a future version could add per-sensor fault latches so that the specific failed channel could be identified rather than only the fact that some fault happened.

## 9.7 Sensor Fusion

Rather than simple majority voting, a more advanced navigation stage could use weighted sensor fusion (for example, a basic Kalman filter) to combine the three navigation channels, which would require a microcontroller or FPGA rather than pure combinational logic.

## 9.8 Artificial Intelligence

A longer-term extension could explore whether a lightweight machine-learning model could improve fault prediction — flagging a sensor as likely to fail before it actually does — though this would be a substantial departure from the deterministic, fully verifiable logic used in this project.

## 9.9 Digital Twin

Building a software digital twin of the mission computer would allow mission scenarios to be tested far more extensively than is practical in Multisim alone, and would let the hardware and simulation be compared side by side during future development.

## 9.10 Hardware-in-the-Loop Simulation

The natural next step after the current Multisim-only verification would be to build the circuit on an actual breadboard or PCB and drive it with a real sensor rig, confirming that the behaviour observed in simulation holds up with physical component tolerances and real switching noise.

# CHAPTER 10: CONCLUSION

This project set out to answer a fairly narrow question — how much of a launch-vehicle mission computer's decision logic could be reproduced using only discrete TTL integrated circuits — and the answer, based on the design and testing carried out here, is that a surprisingly complete subset of that logic can be. The Mission Timer and Sequencer together produced a repeatable, deterministic mission timeline; the Triple Modular Redundancy block demonstrated that a majority-voting scheme can be built from five logic gates and still tolerate a single-sensor failure; and the combination of Decision Logic, Mission Freeze, and Fault Memory showed how a system can move from a momentary fault signal to a persistent, operator-resettable safe state using nothing more than a pair of D flip-flops.

Testing across the nominal and fault-injected scenarios in Chapter 6 confirmed that every subsystem behaved exactly as predicted by its Boolean equations and truth tables in Chapter 4, and the full integrated circuit reproduced the intended mission sequence from Power ON through Payload Deployment without requiring any programmable element.

Beyond the specific circuit, the project gave me a much more concrete understanding of why fault tolerance and deterministic sequencing matter in safety-critical systems, and of how ideas that are usually explained in the context of complex flight software can, at their core, be reduced to a handful of gates and flip-flops. The future-scope discussion in Chapter 9 outlines several directions — FPGA and microcontroller implementations in particular — that would be natural next steps if this project were to be carried further.

# REFERENCES

Ogata, K. (Latest edition). Modern Control Engineering. Pearson Education. [Verify edition/year before final submission]

Mano, M. M., & Ciletti, M. D. (Latest edition). Digital Design. Pearson Education. [Verify edition/year before final submission]

Texas Instruments. (n.d.). 74LS93 4-Bit Binary Counter datasheet. Texas Instruments. [Insert exact retrieval URL/date]

Texas Instruments. (n.d.). 74LS154 4-Line to 16-Line Decoder/Demultiplexer datasheet. Texas Instruments. [Insert exact retrieval URL/date]

Texas Instruments. (n.d.). 74LS74 Dual D-Type Positive-Edge-Triggered Flip-Flop datasheet. Texas Instruments. [Insert exact retrieval URL/date]

Nexperia. (n.d.). 74HC/HCT-series logic datasheets. Nexperia. [Insert exact part numbers/retrieval details actually used]

Indian Space Research Organisation (ISRO). (n.d.). Publications on launch vehicle systems. [Insert specific publication title, year, and URL]

National Aeronautics and Space Administration (NASA). (n.d.). Technical reports on flight computer architecture. NASA Technical Reports Server. [Insert specific report title, year, and URL]

[Additional IEEE reference placeholder — to be completed with a specific paper on TMR / fault-tolerant digital systems relevant to Chapter 2 and Section 4.4]

Note: All datasheet and publication references above are placeholders pending verification of exact edition, year, and retrieval details against the specific sources actually consulted. Please replace bracketed placeholders with verified citation details in APA 7th edition format before submission.

# APPENDICES

## Appendix A — Complete Truth Tables

Refer to Tables 4.1–4.8 in Chapter 4 for the complete set of truth tables covering the Mission Timer, Mission Sequencer, Sensor Health chain, Majority Voter, Launch Authorisation, Mission Abort, Mission Frozen, and Fault Memory subsystems.

## Appendix B — Consolidated Boolean Equations

QA = CLK ÷ 2,  QB = CLK ÷ 4,  QC = CLK ÷ 8,  QD = CLK ÷ 16	(B.1)

Navigation_Health = AB + AC + BC	(B.2)

Sensor_Health = Navigation_Health · Battery · Temperature · Communication	(B.3)

Mission_Launch = Power_ON · Self_Test · Navigation · Sensor_Health · NOT(Mission_Abort)	(B.4)

Mission_Abort = Sensor_Fault + Navigation_Fault + Critical_Failure	(B.5)

## Appendix C — Component List / Bill of Materials

| Item | Component | Quantity | Purpose |
| --- | --- | --- | --- |
| 1 | 74LS93 | 1 | Mission Timer |
| 2 | 74LS154 | 1 | Mission Sequencer |
| 3 | 74LS08 | 3 | AND-based logic (Launch, Sensor Health, TMR) |
| 4 | 74LS32 | 2 | OR-based logic (Majority Voter, Abort) |
| 5 | 74LS04 | 1 | Inverters (Abort logic) |
| 6 | 74LS74 | 2 | Mission Frozen and Fault Memory latches |
| 7 | 74194 | 1 | Telemetry shift register |
| 8 | Toggle switches | 9 | Sensor and control inputs |
| 9 | LED indicators | 5+ | Output display |
| 10 | 5 V TTL clock source | 1 | System clock |

Table D.1: Bill of Materials